DS-3WAP6230-EI
Wi-Fi 6 3000M Ceiling Access Point

DS-3WAP6230-EI is an indoor 802.11ax AP product with dual-frequency four-flow design , which meets the wireless coverage requirements of small, medium and micro enterprises.

- 1 × Gigabit PoE in port,1 × Gigabit RJ45 port

- Wi-Fi 6 Dual Band 3000 Mbps

- Auto RF Optimization

- Wide Signal Coverage

- Reliable Wi-Fi Connection

- Outstanding Wall Penetration

| - Specification |
| --- |
| General |  |
| Net Weight | 0.4 kg (0.88 lb) |
| Gross Weight | 0.64 kg (1.41 lb) |
| Installation Mode | Ceiling Mounting |
| Working Temperature | 0°C to 40°C (32°F to 104°F) |
| Working Humidity | 10% to 90% (no condensation) |
| Storage Temperature | -40°C to 70°C (-40°F to 158°F) |
| Storage Humidity | 5% to 95% (no condensation) |
| Product Dimensions (W × H × D) | Ø220 mm × 49.7 mm (excluding bracket） |
| Packing List | 3000 M Access Point × 1, Screw Pack × 1, Installation bracket × 1, Quick Start Guide × 1, Regulatory Compliance and Safety Information × 1 |
| Power Supply Method | DC power supply: 12 V, 2 A; PoE power supply: IEEE 802.3at; |
| Max. Power Consumption | 18 W |
| Wireless Parameter |  |
| Wireless Speed | Max: 2976 Mbps; 2.4G: 574 Mbps; 5G: 2402 Mbps; |
| Wireless Standard | Wi-Fi 6 |
| Working Frequency Band | Radio 1: 5G (11a/n/ac/ax); Radio 2: 2.4G (11b/g/n/ax); |
| Bandwidth | 2.4G: Auto/20/40 MHz; 5G: Auto/20/40/80/160 MHz; |
| Recommended Coverage Range | Recommended Coverage Range: 140 m², the radius in an open environment is 25 meters. Note: The signal coverage range is greatly affected by the environment. For example, walls, glass doors, and other obstacles will affect the signal coverage. The above - mentioned values are for reference only. |
| Max. Number of Connected Clients | 256 |
| Recommended Number of Connected Clients | 110 |
| Spatial Streams | 2.4G: 2 × 2, MU-MIMO 5G: 2 × 2, MU-MIMO |
| Transmitting Power | 2.4G: Combined power: 28 dBm; 5G: Combined power: 30 dBm; Note: The actual transmission power varies according to regulations in different countries and regions; |
| Wireless Security | WPA2-PSK, WPA-WPA2-PSK, WPA3-PSK, WPA2-WPA3-PSK |
| Antenna Type | Built-in omnidirectional antennas |
| Antenna Gain | 2.4 GHz: 3.5 dBi 5 GHz: 4.5 dBi |
| Hardware Specifications |  |
| Ports | 1 × Gigabit PoE in port, 1 × Gigabit RJ45 port |
| RESET | 1 × Reset hole |
| Software Specifications |  |
| Max. Number of Wireless SSIDs | 8 |
| Internet Access Mode | DHCP,Static |
| VLAN ID Range | 1~4000 |
| Max. Number of VLANs | 8 |
| Software Features |  |
| Operation Mode | AP mode (default); Routing mode; |
| Wireless Management | Support wireless configuration; Support RF configuration; Support Health Mode; Support guest SSID; Support wireless roaming; Support wireless optimization; Support wireless SSID speed limit; Support wireless RF speed limit; Support wireless intelligent speed limit; Support wireless terminal speed limit; Support weak signal access control; Support weak signal to kick offline; Support roaming sensitivity adjustment; Support wireless access control; Support wireless terminal association; |
| Interface | Support uplink port VLAN; Support MAC address modification; Support LAN configuration; Support Port VLAN; |
| IP Services | Support DHCP Client; Supports DHCP Server; Support LLDP; Support SADP; |
| Service and Support | Support NAT Traversal; Support Audit Log; Support Fault Diagnosis; Support Fault Alarm; Support One-Click Fault Collection; Support Network Diagnostic Tools; Support SSH; Support Cloud Service Diagnosis; Support Local Upgrade; Support Online Upgrade; Support Cloud Service; Support App Management; |
| Approval |  |
| EMC | CE-EMC (EN 55032:2015/A1:2020 EN 55035:2017/A11:2020 EN IEC 61000-3-2:2019/A2:2024 EN 61000-3-3:2013/A2:2021 EN 50130-4:2011/A1:2014) |
| Safety | CB (IEC 62368-1: 2018), CE-LVD (EN IEC 62368-1: 2020+ A11: 2020) |
| Chemistry | CE-RoHS (2011/65/EU), WEEE (2012/19/EU), Reach (Regulation (EC) No.1907/2006) |
| Radio | EN 300 328, EN 300 440, EN 301 893, EN 301 489-1, EN 301 489-3, EN 301 489-17, EN 61000-3-3: 2013+ A1:2019+A2: 2021, EN IEC 61000-3-2: 2019+A1: 2021+A2: 2024, EN IEC 62311: 2020 |

- Available Model DS-3WAP6230-EI

- Dimension